1Software Implementation Of

Asynchronous Serial I/O Using PIC16Cxx

Introduction

The PIC16Cxx series from Microchip Technology, Inc., are 2nd generation high performance EPROM based 8 bit microcontrollers. Some of the members of this series (like PIC16C71 & PIC16C84) do not have on chip hardware asynchronous serial port. This application note describes the Interrupt driven Software implementation of Asynchronous Serial I/O (Half Duplex RS-232 Communications) using PIC16CXX microcontrollers. These microcontrollers can operate at very high speeds with a minimum of 250 nS cycle time (with input clock freqency of 16 Mhz). To test the RS-232 routines, a simple Digital Volt Meter (DVM)/Analog Data Aquisition Systems has been implemented using PIC16C71 in which upon reception of a commnad from host (IBM PC),  an 8 bit value of the selected A/D channel is transmitted back to host.

Implementation

A half duplex Interrupt driven software implementation of RS-232 communications using PIC16C71 is described in detail below. The transmit pin used in the example code is RB7 and receive pin is connected to RTCC/RA4 pin. Ofcourse these pin are connected with appropriate voltage translation to/from RS-232/CMOS levels. The voltage translation is given described with schematics in the hardware section of this application note.

Transmit Mode :

The transmit mode in software is quite straight forward to impelment using interrupts. Once the input clock frequency  & baud rate is known, the number of clock cycles per bit can be computed. The on chip Real Time Clock Counter (RTCC) along with the prescaler can be used to generate interrupt on RTCC overflow. This RTCC overflow interrupt can be used  as timing to send each bit. In this application note, the Input clock frequency ("_ClkIn")  and the Baud Rate ("_BaudRate") are programmable by the user and the RTCC timeout value (the period for each bit) is computed at assemble time. Whether the prescaler must be assigned to RTCC or not is also determined at assembly time. This computation is done in the header file "rs232.h". Note that very high speed transmissions can be obtained if transmission is done with pulery software delays instead of interrupt driven. However the processor will be totally dedicated  to ths job.

Transmission of a byte is performed by calling "PutChar" function and the data byte in "TxReg" is transmitted out. Before calling this function ("PutChar"), the data must be loaded into TxReg and also made sure that serial port is free. The serial port is free when both _txmtProgress and _rcvOver bits are cleared (see description of these bits in the Serial Status/Control Reg table given later ).

Summary of "PutChar" function :

1) Make sure _txmtProgress & _rcvOver bits are cleared

2) Load TxReg with data to be transmitted

3) CALL  PutChar function

Receive Mode :

The reception mode implementation is slightly different from the transmit mode. Unlike the transmit Pin (TX pin in the example code is RB7, but could be any I/O pin), the receive pin (RX Pin) must be connected to RTCC/RA4 Pin. This is because in reception, at first the Start Bit must be detected which is asynchronous in nature. To detect the start bit, when put in Reception mode, the RTCC module is configured to counter mode . The OPTION register is configured so that RTCC module is put in counter mode(increment on external clock on RTCC/RA4 Pin) and set to increment on falling edge on RTCC/RA4 pin with no prescaler assigned. After this configuration setup, RTCC (File Reg 1) is loaded with 0xFF. A falling edge on RTCC Pin will make RTCC roll over from 0xFF to 0x00, thus generating an interrupt indicating a Start Bit. The RTCC/RA4 pin is sampled again to make sure the transition on RTCC is not a glitch. Once the start bit has been detected, the RTCC module is reconfigured to increment on internal clock and t

The software serial port is put in reception mode when a call is made to function "GetChar". Before calling this function make sure serial port is free (i.e. _txmtProgress and _rcvOver status bits must be 0). On completion of reception of a byte, the data is stored in RxReg and _rcvOver bit is set to 0.

Summary of "GetChar" function :

1) Make sure _txmtProgress & _rcvOver bits are cleared

2) CALL GetChar function

3) The received Byte is in TxReg after _rcvOver bit is cleared

Parity Generation :

Parity can be enabled at assembly time by setting "_PARITY_ENABLE" flag to TRUE. If enabled the parity can be set to either EVEN or ODD parity. Intransmission mode, if parity is enabled, the parity bit is computed and transmitted as the 9th bit. On reception, the parity is computed on the received byte and compared to the 9th bit received. If a match does not occur the parity error bit is set in the RS-232 Status/Control Register ( _ParityErr bit of SerialStatus reg). The parity bit is computed using the algorithm shown in Fig. A  This algorithm is highly efficient using PIC16Cxx's SWAPF and XORWF instructions (with ability to have the destination as either file register itself or W register) and the sub-routine (called "GenParity" ) is in file "txmtr.asm".

EMBED MSDraw   \* mergeformatµ §

Fig A : An Efficient Parity Generation Scheme In Software

Assembly Time Options :

The firmware is written as a general purpose routines and the user must specify the following parameters before assembling the program. The Status/Control register is also described below:

_ClkIn

Input clock frequency of the processor

_BaudRate

Desired Baud Rate. Any valid value can be used. The highest Baud Rate achievable depends on Input Clock Freq. 600 to 4800 Baud was tested using 4 Mhz Input Clock. 600 to 19200 Baud was tested using 10 Mhz Input Clock

Higher rates can be obtained using higher Input Clock Frequencies.

Once the _BaudRate & _ClkIn are specified the program automatically selectes all the appropiate timings

_DataBits

Can specify 1 to 8 data bits

_StopBits

Limited to 1 Stop Bit. Must be set to 1

_PARITY_ENABLE

Parity Enable Flag. Set it to TRUE or FALSE. If PARITY is used, then set it to TRUE, else FALSE. See "_ODD_PARITY" flag description below

_ODD_PARITY

Set it to TRUE or FALSE. If TRUE, then ODD PARITY is used, else mEVEN Parity Scheme is used.

This Flag is ignored if _PARITY_ENABLE is set to FALSE.

_USE_RTSCTS

RTS & CTS Hardware handshaking signals. If set to FALSE, no hardware handshaking is used. If set to TRUE, RTS & CTS use up 2 I/O Pins of PortB

Fig 2.1	List Of Assembly Time Options

Bit #

Name

Description

0

_txmtProgress

1 Indicates Transmission in progress. A 0 indicates transmission line free. When a Byte

1

_txmtEnable

Set this bit to 1 on initialization to enable transmission. This bit may be used to abort a transmission. The transmission is aborted if in the middle of a transmission (i.e. when _txmtProgress bit is 1) _txmtEnable bit is set to 0. This bit gets automatically set when PutChar function is called

2

_rcvProgress

This bit is 1 if in the middle of a byte reception. It is reset to 0 when reception of a byte (in RxReg) is complete and is set to 1 when a valid start bit is detected in reception mode.

3

_rcvOver

A 1 indicates the completion of reception of a byte. The user's code can poll this bit after calling "GetChar" function and check to see if it is set. When set, the received byte is in RxReg. Other status bits should also be checked for any reception errors.

4

_ParityErr

A 1 indicates a parity error on reception (irrespective of Even Or Odd parity chosen). Not applicable if No Parity is used.

5

_FrameErr

A 1 indicates a framing error on reception.

6

Unused

7

_parityBit

The 9 th bit of transmission or reception. In transmission mode, the parity bit of the byte to be transmitted is set in this bit. In receive mode, the 9th bit (or parity bit) received is stored in this bit. Not Applicable if no parity is used.

Fig 2.2	Bit Assignments of  Serial Status/Control Reg ("SerialStatus" reg)

Hardware :

The hardware is primaraly concerned with voltage translation from RS-232 to CMOS levels and vice versa. Three circuits are given below and the user may choose which ever best suits his application. The primary difference between each solution is cost versus number of components. Circuits in Fig 2.2 & 2.3 are very low cost but have more components than the curcuit in Fig 2.1. The circuit in Fig 2.1 interfaces to RS-232 line using a single chip (MAX-232) and single +5V supply. The circuit in Fig 2.2 is a low cost RS-232 Interface but requires 2 chips and a single +5V supply source.

Fig 2.3 shows a very low cost RS-232 Interface to an IBM PC  with no external power requirements. The circuit draws power from RS-232 line (DTR) and meets the spec of drawing power less than 5mA. This requires that the host to communicate must assert DTR high and RTS low. The power is drawn from DTR line and this requires that DTR to be asserted high and must be atleast 7V. The negative -5 to -10 V required by LM339 is drawn from RTS line and thus the host must assert RTS low. This circuit is possible because of the low current consumption of PIC16C71 ( typical 2 mA ).

Fig 2.1 Single chip, Single +5V RS-232 Interface

Fig 2.2 Low Cost RS-232 Interface (2 chip, single +5V supply)

Fig 3 Low Cost RS-232 Interface (Power Drawn From RS-232 Lines)

Test Program :

To test the transmission & reception modules, a main program is written in which the PIC16C71 waits to receive a command from a host thru the RS-232. On reception of a byte (valid commands are 0x00, 0x01, 0x02 & 0x03),  the received byte is treated as the PIC16C71's A/D channel number and the requested channel is selected, an A/D conversion is started and when the conversion is complete (in about 20 uS) the digital data (8 bits) are transmitted back to the host. A Microsoft Windows program running on an IBM PC/AT was written to act as a host and collect the A/D data from PIC16C71 via an RS-232 port. The Windows program (DVM.EXE) runs as a background job and displays the A/D data in a small window (similar to the CLOCK program that comes with MS Windows). The windows program and the PIC16C71 together act like a data aquisition system or a digital volt meter (DVM). The picture of the whole system is shown in Fig 2.1. The input clock frequency is fixed at 4 Mhz and RS-232 parameters are set to 1200 Baud, 8 Bit

Fig 3.0 MS Windows Program Fetching A/D Data From PIC16C71 Via RS-232

Source Code :

The PIC16Cxx source code along with the MS Windows DVM Program (executable running on an IBM PC/AT under MS Windows 3.1 or higher) is available on a floppy disk. The assembly code for PIC16Cxx must be assembled using Microchip's Universal Assembler MPASM. The code cannot be assembled using the older assemblers without significant modifications. It is suggested that user's who do not have the new assembler MPASM, must change to the new version.

The MS Windows Program (DVM.EXE) runs under MS Windows 3.1 or higher. The program does not have any menus and shows up as a small window displaying A/D Data and runs as a background job. There are a few command line options and are described below :

-Px	:	x is the comm port number (e.g -P2 selects COM2). Default is COM1

-Cy	:	y is the number of A/D channels to display. Default is one channel(channel #1)

-Sz	:	z is a floating point number that represents the scaling factor  (For example -				S5.5 would display the data as 5.5*<8bit A/D>/256). The default value is 5.0 				volts. -S0 will display the data in raw format without any scaling.

„-z55:55ü4Å<55555555555555
-z

èè5555555555555555555555 5	55	555Z55555§555&¤55ÿÿÿÿ5§555MrEd¤5Microsoft Draw55Z555&¤5©5MrEd5Ÿ5555 5§55óš555555 555555ÿÿÿ55555ÿÿÿ5 55 555555555 55

C6

7System77§7ÿ7ð   7P77á7ÿ7ñ 77777ÿÿÿ7ÀÀÀ7€€€7ÿ777€777ÿÿ77€€777ÿ777€777ÿÿ77€€777ÿ777€7ÿ7ÿ7€7€77

888&¤88MrEd8888hÆ8l

*999
Æ9h999

d