SXSIM.DOC        AUG-09-99

SxSim is a simulator for the Scenix SX18, SX20 and SX28 microcontrollers. If

you've used Parallax's PSIM, SxSim should look familiar. It runs under DOS

and it's easy to use.

SxSim requires a .LST file generated by either Parallax's SX-Key or SPASM,

Microchip's MPASM, or Scenix's SASM assembler. Start the simulator by

typing: SXSIM MYFILE, where MYFILE is the name of your .LST file. The ".LST"

is assumed and need not be typed.

When SxSim starts, it simulates a power-on reset. Most of the registers come

up in a random state.

The spacebar is used to single-step through a program to see how it works.

If you want to take a closer look at some steps, back up using the backspace

key and step through them again. If you want to run the program, press F9.

The simulation will go until a key is struck or a breakpoint is reached.

The changing colors on the top half of the screen show the action as your

program executes. When a number changes, it's momentarily displayed in white

to draw your attention. The green background points out the registers and

status bits used by the instruction about to be executed.

The W register is displayed on the left side. Its contents are shown in hex,

ASCII and binary. The individual bits of the status register are shown to

the right of the W register. The I/O ports (RA, RB, RC) are shown both in

hex (on the left) and in binary (on the right).

There are two kinds of cursors in the listing window. The green one shows

the location of the program counter (PC), and the black one is the line

cursor. You use the line cursor to move through the code and to set break-

points, which are red.

Here's a list of all the commands:

F1  Help    Show this help screen

F2  Bkpt    Set or clear breakpoint at line cursor (toggles)

F3  Clear   Remove all breakpoints

F4  Here    Execute code until PC reaches line cursor

F5  Time    Reset elapsed time to 0.000 seconds

F6  Go      Execute code, and display result for each instruction

F7  Step    Execute one instruction

F8  Over    Same as F7 but skip over CALLs (run them at full speed)

F9  Run     Execute code at full speed (until breakpoint)

F10 MClr    Simulate master clear (hardware reset)

Alt:

F1  Output  Show screen used for OUTPUT instruction

F3  Count   Show number of times instructions have been executed

F4  Jump    Set PC to line-cursor location (without executing)

F6  Slow    Execute and display at slow speed (Go Slow)

F7  Back    Undo last instruction (step backward)

F8  Return  Execute until return to caller (skip rest of CALL)

F10 Reset   Simulate master clear, and zero registers

Spacebar    Execute one instruction (same as F7 and Enter keys)

Backspace   Undo last instruction (same as Alt+F7)

^ v         Up and down arrows move line cursor up or down a line

Page Up/Dn  Move line cursor up or down a page

Home        Move line cursor to location 000

End         Move line cursor to last location

Mouse       Left button:  Increment value / Set or clear breakpoint

Right button: Decrement value / Execute up to here

Esc         Exit back to operating system

MOUSE

The mouse is used to change values in registers. The left button increments

the digit under the cursor, and the right button decrements it. Holding the

button auto-repeats. An important use of the mouse is to change the inputs

on the port pins. The values for the IND (indirect) register and the WDT

(watchdog timer) cannot be changed with the mouse. Clicking on the time

resets it to 0.000.

The oscillator frequency can be adjusted with the mouse. To change megahertz

to kilohertz, click on the "M" with the right mouse button. Changing the

frequency, of course, changes the time it takes to execute instructions. The

frequency ranges from 1.0 kHz to 1.0 GHz.

In the listing window, pressing the left mouse button sets or clears a

breakpoint. Pressing the right mouse button executes code up to the line the

mouse cursor is on.

The commands listed at the bottom of the screen can be executed by clicking

on them or by pressing the function keys. Clicking on "F7=Step" with the

right mouse button steps backwards instead of forwards. The [�] symbol in

the upper-left corner is the program exit button.

SASM

There is a small problem with the .LST files produced by Scenix's assembler,

SASM 1.39. The "device" directive prints the FUSE and FUSEX values in the

address and opcode fields of the listing. SxSim can get very confused by

these. The problem is easily solved by inserting a label. For some reason

this prevents these fuse values from being listed. (This has no effect on

the .HEX file, which has the correct fuse values either way.) For example,

the dummy label FUSE is added here:

FUSE    DEVICE  PINS28,PAGES1,BANKS8,OSCHS,TURBO,STACKX,OPTIONX

The SX-Key assembler from Parallax does not allow a label for the "device"

directive. You will need to modify your code depending on which assembler

you use.

SX-KEY.LST

Getting a listing file from the SX-Key assembler isn't obvious. Click on

"List Toggle" under the "File" menu to display a listing of your program.

Again under the "File" menu, click "Save As..." and after "File name:"

enter: MYFILE.LST, where MYFILE is the name of your program. If you're using

the SX-Key editor prior to version 1.07, it might append a ".SRC" extension

that must be removed. The command to do this under DOS is:

RENAME MYFILE~1.SRC MYFILE.LST.

FUSE BITS

The configuration registers, FUSE and FUSEX, are normally set like this:

FUSE    FUSEX

For SX:     0FB     FFE

For PIC:    FFB     FFA

These values are saved in a file with the same name as your .LST file but

with a .CFG extension. You can edit this file directly or use the mouse to

change these values on the screen. If the .CFG file is not found then the

fuses default to the SX settings (0FB & FFE). Fuse bits are set up auto-

matically if the DEVICE directive in SXDEFS.INC is used with MPASM. Fuse

bits are not set up automatically by other assemblers. The fuse bit

assignments for the new 'A'-style parts (date code Axyywwxx) are not

simulated.

The size of the instruction memory is determined by the highest location

used in the .LST file. The size can be either 512, 1K, 2K, or 4K words.

FUSEX is automatically set to indicate the size of the memory actually used.

SxSim simulates the following fuse bits:

FUSE

11    10     9     8  |  7     6     5     4  |  3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+

|/Trbo|  -  |/OptX|/StkX|/IRC |Div2 |Div1 |Div0 |  -  |WDTE |  -  |  -  |

+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+

/Trbo   0 = Turbo mode is enabled (instruction clock = OSC/1)

1 = Turbo mode is disabled (instr. clock = OSC/4) (PIC compatible)

/OptX   0 = OPTION register is extended to 8 bits (RTCC/W select, RTCC int.)

1 = OPTION register uses only 6 bits (PIC compatible)

/StkX   0 = 8-level STACK

1 = 2-level STACK (PIC compatible)

/IRC    0 = Internal RC oscillator is enabled

1 = Internal RC oscillator is disabled

Div2:Div0   Internal RC oscillator divider:

000 = 4.0 MHz

001 = 2.0 MHz

010 = 1.0 MHz

011 = 500 kHz

100 = 250 kHz

101 = 125 kHz

110 =  62.5 kHz

111 =  31.25 kHz

WDTE    0 = Watchdog timer is disabled

1 = Watchdog timer is enabled

FUSEX

11    10     9     8  |  7     6     5     4  |  3     2     1     0

+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+

|  -  |Pins |  -  |  -  |  -  | /CF |  -  |  -  |RAM1 |RAM0 |Mem1 |Mem0 |

+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+-----+

Pins    0 = 18-pin package, location 07 = RAM, Port RC is disabled

1 = 28-pin package, location 07 = Port RC

/CF     0 = Carry input for ADD and SUB instructions is enabled

1 = Carry input is disabled (PIC compatible)

RAM1:RAM0   Enabled number of RAM banks:

00 = 1 bank

01 = 2 banks

10 = 4 banks

11 = 8 banks

Mem1:Mem0   Enabled number of words (automatically set):

00 = 1 page  (512)

01 = 2 pages (1K)

10 = 4 pages (2K)

11 = 8 pages (4K)

MISCELLANEOUS

In addition to the fuse bits the .CFG file also contains the oscillator

frequency (in Hertz) and the watchdog timeout period (in seconds). The only

way to change the watchdog timeout period from the nominal 18 milliseconds

is to edit this file.

SxSim is based on documentation from both Scenix and Microchip with priority

given to Microchip's documentation for the Enhanced PIC16C5x. This shouldn't

be a problem for Scenix parts since the SX is a superset of the 16C5x. This

also assures a correct simulation of the 16C5x when the FUSE bits are

configured for compatible mode.

The value displayed for I/O ports is the value on the pins. This is the

value your program sees when it reads a port. A value written to a port is

written to its latch. What actually appears on the pins depends on the

tristate (data direction) register.

The SLEEP instruction is correctly simulated when the WDT is enabled, but it

works like a breakpoint when the WDT is disabled.

If you're using Microchip's MPASM, the new instructions, such as RET and

IREAD, can be replaced by macros or DATA instructions (see SXDEFS.INC).

Unfortunately Parallax's SPASM doesn't have these features, and the only

alternative is to modify the opcodes directly in the .LST file using a text

editor. If you use macros, don't use NOEXPAND since SxSim relies entirely on

the .LST file for its input. Also be careful how you use NOLIST.

SxSim recognizes three instructions that aren't in the microcontroller chip.

The INPUT instruction (00Ah) waits for a keystroke then sets the W register

to its ASCII value. The Esc key is used to restore normal operation of the

simulator. The OUTPUT instruction (00Bh) displays the character correspond-

ing to the value in the W register on an alternate screen. Use Alt-F1 to

view this screen. The PRINT instruction (001h) prints the value in the W

register on your printer. These instructions can be inserted using the DW

directive, for example: DW 0Bh. SPASM doesn't have a DW directive, and the

only alternative is to enter the opcode directly in the .LST file.

Instructions leave tracks by displaying a dot after the address once they're

executed. This is useful when you want to verify that all paths through a

program have been tested. Similarly, file registers that have been written

to (or initialized) are also marked with a dot.

Cycle counts are not displayed. If you prefer to think in terms of cycle

counts rather than time, then select an oscillator frequency of 1.000 MHz.

This makes each cycle take exactly one microsecond (in turbo mode), and the

TIME display can be thought of as a cycle counter. If you're simulating a

PIC, which divides the oscillator frequency by four, then set the frequency

to 4.000M.

SxSim automatically stretches to the size of the display. I like to run the

program L36, which gives 36 lines with a nice clean font, before running

SXSIM. L25 gets you back to the standard 25-line display. The batch file

SS.BAT does all this automatically.

STIMULUS FILES

Input pins not only can be changed with the mouse, but they also can be

changed using a stimulus file. A stimulus file specifies the inputs and when

they occur. Here's an example of a command line in a stimulus file:

T=5E-6 RA=3      ;Comment

This sets port RA to 3 (0011b) after the program has been running for a

simulated 5.0 microseconds. (5E-6 = 0.000005 seconds.)

The format is similar to that used by PSIM. Numbers must be terminated by a

non-numeric character such as a space or comma. Either upper or lower case

characters can be used. Comments are preceded by a semicolon, just like in

assembly code.

The other ports, RB and RC, also can be set. The port value is always in

hex. For example: RB=f3. More than one port can be set on the same line, and

individual bits can be set or cleared like this: RC5=1, rb2=0. The RTCC pin

can be set or cleared with: RTCC=1 or RTCC=0.

Breakpoints also can be set at a specified time. Simply add the word "break"

after the time. (Actually any word beginning with an upper or lower case "b"

will do.) For example: "T=1 rtcc=1 BP" sets RTCC high and halts after one

second.

Time also can be specified as an interval (or delta) since the last command.

For example: dt=.001 rb7=1 sets the high bit of RB one millisecond after the

previous command.

When SxSim starts, it searches for a stimulus file. If it finds a file with

a .STM extension and a name matching the input (.LST) file, it loads it as

the stimulus file. For instance, HELLO.STM is the stimulus file for

HELLO.LST. If the file is not found then a stimulus file is not used.

CHECKPOINTS

You can save the state of your simulated program at any time using a

checkpoint. Everything is saved to a disk file, including registers,

breakpoint locations, and the history memory used by the backup command.

To save a checkpoint press Shift+Fn, where Fn is one of the function keys F1

through F10. The saved information can be restored later by pressing

Ctrl+Fn.

For example, you might have a program with a delay loop that takes many

seconds to reach the code you're trying to test. When you reach this code,

if you press Shift+F3, you can immediately return to this point by pressing

Ctrl+F3. This lets you test several scenarios without having to wait while

the program runs through the delay loop each time.

You can even exit the simulator, make a modification to your code, restart

the simulator, then press Ctrl+F3 to immediately get back to where you were.

However, there's a danger that breakpoints can shift on you if you change

the number of instructions. Things can really fall apart fast if your

modifications are extensive, or if you try to back through them using the

backspace key. Also, please don't try to load a checkpoint from an earlier

version of SxSim.

WINDOWS

SxSim is intentionally designed as an alternative to Windows-based programs

such as Parallax's SX-Key or Microchip's MPLAB. If you run SxSim under

Windows 95 or 98 and you're concerned about speed, you might consider

trading in your Pentium for a 286. SxSim runs an unbelievable 35 times

slower under Windows than it does under pure DOS (not in a DOS box under

Windows). Fortunately, you can greatly speed it up (about 2/3 the DOS speed)

by doing the following:

Run SxSim in a window (Alt+Enter flips between full screen and a window).

Click on the MSDOS icon in the upper-left corner of the window.

Select Properties.

Select Misc.

Move the Idle sensitivity slider all the way to Low.

Click OK.

Hit Alt+Enter to get back to a full screen.

Another little problem with Windows is that it alters the colors so that

white characters don't show up well on the green background. This is solved

by running in full-screen mode.

PROBLEMS

Here's a list of known problems and deficiencies. (Dare I say, BUGS?) If you

discover a problem not listed here, I'd definitely like to hear from you.

The SX48/52 parts are not simulated.

The fuse bit assignments for the new 'A'-style parts (date code Axyywwxx)

are not simulated.

Labels should not be on a line by themselves since lines without instruction

mnemonics are not shown in the listing window. Everything operates okay; you

just can't see the labels. (PSIM has this problem too.)

The backup command (backspace key) does not restore registers changed with

the mouse. Pin inputs are not restored whether changed by the mouse or by a

.STM file. Stimulus files do not backup.

INCFSZ PC takes 2 or 3 cycles in PIC-compatible mode and 2 or 4 cycles in

turbo mode. Since this is a very unlikely instruction, I haven't measured

the actual cycles, but these are probably wrong.

Analog signals such as TTL and CMOS levels, pullups, Schmitt-triggers, and

the comparator on port RB are not simulated.

The following problems can occur when using MPASM:

If a title looks like an instruction, it will cause problems. For example,

this loads 004 into location 0BE: TITLE "0Be 04 Confuses SxSim"

The __CONFIG instruction can cause a problem. For a 16C58A, __CONFIG 0123h

makes the simulator think there is 4K of instruction memory (instead of 2K)

and loads location 0FFF with 0123.

LICENSE

This program is free software; you can redistribute it and/or modify it

under the terms of the GNU General Public License version 2 as published by

the Free Software Foundation.

This program is distributed in the hope that it will be useful, but WITHOUT

ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or

FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for

more details.

You should have received a copy of the GNU General Public License along with

this program (in the file LICENSE.DOC); if not, write to the Free Software

Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

If you have any comments, suggestions, or bug reports, I'd really like to

hear from you. You can reach me at:

Email: loren_blaney@idcomm.com     Regular Mail: 502 Pine Glade Dr.

Nederland, CO 80466

The latest version can be downloaded from:       USA

http://www.brouhaha.com/~eric/scenix/sxsim/

Thanks for your interest in SxSim.

-Loren Blaney